A PRIMAL PARTITIONING APPROACH TO THE MINIMUM COST MULTICOMMODITY NETWORK FLOW PROBLEM

ABSTRACT

This work presents a partitioning-based simplex algorithm to solve the minimum cost flow problem on multicommodity networks. Such partitioning approach reduces the size of the working-basis down to the number of saturated arcs. A working basis is built using lines related to flow constraints of saturated arcs, and columns related to path-variables of split-flow commodities . These columns and lines define a “cycle matrix”. Each iteration updates the cycle matrix inverse, attaching a new pivoting matrix to a product form representation. As the number of saturated arcs changes, there is also an update in the size of the cycle matrix; this size changing operation is performed adding non-square elementary matrix to both sides of the pivoting chain.

# Introduction

The Minimum Cost Multicommodity Flow Problem (MCMFP) has been intensively studied [1, 4, 7, 9, 10, 12, 13, 14], given its applicability on many network synthesis problems found in telecommunications and transportation industries. Many researchers have already developed specialized versions of SIMPLEX and interior-point algorithms to solve MCMFP problems [5, 12, 13, 14]. Why?

Large MCMFP problems may contain hundreds of nodes and thousands of arcs. These problem sizes prohibits using traditional SIMPLEX algorithms in personal computers, since the basis-inversion demands too much RAM memory. There is no algorithm similar to the network-simplex long used in mono-commodity flow problems.

The MCMFP can be modeled through node–arc or arc–chain formulations. Each one has its advantages and drawbacks. The node-arc formulation models the problem by assuring flow-equilibrium for each commodity throughout all network nodes. These flow-equilibrium equations define a set of incidence matrix Ap, one for each commodity p . A

second group of constraints assures that arcs will not be used beyond their maximum capacity. Figure 1 describes the matrix structure of this formulation.

Figura I-1: matrix structure of a minimal cost multicommodity network flow problem using a node-arc representation

Such a node-arc approach does not require a previous route-finding step. However it is harder to model routing-costs and routing-constraints using it. One can find out how hard it to incorporate the following features in this model:

- path-associated costs, i.e., each path has its own costs.

- constraints based on maximum number of spans in each route.

Another drawback is the astonishingly large size of the linear programming (LP) problem associated to the MCMFP on large networks.

As an alternative approach, arc–route formulation assures that each commodity is routed over a set of pre-calculated bundle of routes. This approach may deviates the solution from an equivalent node-arc global optimum, since optimal node-arc routes may be outside the candidate routes set. However, it is still preferred by planners in many situations. For example, it is well suited to telecommunications routing problems where technological limitations refrain from using routes with more than a maximum number of arcs. Such a situation occurs when impedance problems have to be taken into account.

However, its great advantage it that it generates smaller LP models (see Figure I-2), once guaranteed a reduced number of candidate paths may be used. This guarantee if feasible, since optimal node-arc solutions always use a small number of routes. In fact, linear programming theory permits us to show that at most k + m paths carry positive flow in some optimal solution to a MCMCP problem [10], where k = number of commodities and m = number of saturated arcs. Ford and Fulkerson (1958) observed that a relatively small number of paths will dominate the majority. This fact has inspired the development of path-generation schemes to obtain global optimal solution even using arc-chain formulations[13].

Figura I-2: (2) matrix structure of a minimal cost multicommodity network flow problem using arc-path formulation

Both formulations have special structures that allow the use of Generalized Upper Bounding (GUB) techniques for their solution [2, 3, 6, 8, 10, 11]. We stress that this work will focus arc-path formulations only.

The traditional GUB technique reduces of the working-basis dimension to the number of network arcs. It also classifies commodities in two groups (accordingly to Lasdon, in [6]):

- essential commodities:  those which have their flow split in two or more different paths, and

- non-essential commodities: those which have all their flow routed through a single path .

Each commodity is bound to use at least one path (the key path), thus dividing paths in key- and non-key-paths. Arcs are also classified as saturated and non-saturated arcs, depending on how they are used (completely or with an unused slack). These concepts (key, non-key, essential, non-essential, saturated or non-saturated) will be used extensively in this work.

In [12], Kennington and Helgason (K&H) proposed a product-form representation of the multicommodity cycle matrix. A cycle matrix is obtained by column- and line-operations over  non-key columns of essential commodities. The  cycle-matrix inverse is then used to obtain the GUB working-basis inverse in a node-arc formulation.  In the same paper, K&H proposed an ingenious updating process to obtain the cycle matrix inverse at each iteration based upon the preceding iteration one.

More recently, Farvolden et alii [13] proposed an algorithm to solve arc-route formulations using a similar cycle-matrix. However, this later work does not updates the working-basis while going from one basis to the following one. Each iteration requires solving a (expectedly) small linear system of equations. In the same paper, Farvolden proposed further studies in re-factorization methods to make this step computationally cheaper.

In this paper we propose a SIMPLEX specialization to solve the minimum cost multicommodity flow problem in the same way as Kenington & Helgason, and very similar to and Farvolden et alii. We use a product-form representation of the cycle matrix to  update the inverse from one iteration to the next one. As well as Farvolden did, we also show that all remaining calculations can be interpreted as network flow operations. The algorithm is applied to a network design  example, to make it easier to comprehend.  The algorithm main advantage is the economic utilization of RAM memory (once guaranteed a small number of saturated arcs), making it suitable to be implemented in relatively small computers even to solve very large real problems.  A computational implementation was being produced when this technical report was released.

Chapter 2 describes the arc-chain formulation of the MCMFP problem and discusses how large an MCMFP working-basis can be in different formulations.  Chapter 3 presents  the basis partitioning adopted in this work and how to calculate the cycle matrix Q. Chapter 4 discusses how to use the inverse of Q to calculate dual variables, primal variables and updated columns in each SIMPLEX iteration. Chapter 5 shows how to update Q-1 from iteration i, so that it can be used in iteration (i+1). Chapter 6 presents an example of application of the proposed algorithm.

# Arc-chain formulation  & problem size

This chapter presents the Arc-Chain formulation as an alternative to model the MCMF problem. Here we also compare the size of the working-basis using different partitioning approaches.

## The MCMF Problem using the Arc-Chain Formulation

Let Pk = R1k,...,Rnkk be the set of nk paths connecting source and sink of commodity k. We may describe these routes using an arc – chain incidence matrix, each entry defined as follows:

aij=1,ifarcaibelongtochainRj0,otherwise

In order to work easier with multi-commodities, we will change slightly this definition to a more flexible one:

aij,k=1,ifarcaibelongtochainRjk,thatisthej−throuteofcommodityk,0,otherwise

Using this definition, we can define the Minimum Cost Multicommodity Flow (MCMF) problem as:

MinimizeCj,kxj,k							(1)

subjecto to

aij,kxj,k≤bi		 i = 1, ..., m			(2)

xj,k=dk			 k = 1, ..., p			(3)

xj,k≥0				 j = 1, ..., nk,  k = 1, ..., p	(4)

where:

xj,k	is the amount of commodity k  flown through route Rjk

Cj,k	is the cost to transport one unit of commodity k through route Rjk.

bi	is the maximum flow on arc i.

dk	is the total amount of commodity k to be transported.

Constraints 1-3 can be read as follows:

- distribute dk over some of the routes available to commodity k in order to minimize overall cost. Do it for all k = 1, ..., p, where p is the total number of commodities in the network;

- each arc ai, i = 1, ..., m, is constrained to have less than or equal to a maximum flow bi;

- each commodity k has an amount of dk units to be transported from origin to destination.

Constraint 2 may be enhanced to consider network design features, such as:

- each arc capacity may be upgraded, buying/building more transport facilities. The amount of facilities is represented by variable yi, with unit-cost equal to i.

- constant bi can be considered as installed facilities, with zero-cost.

- New facilities are usually installed in large modules that may be not used completely. Although this “integer decision variable” will not be considered here, the algorithm described in this paper is being prepared to act as sub-problem-solver to integer network design algorithms. Then, these slacks would have to be considered. Anyway,  even zero-cost facilities may be left unused. These are reasons enough to include slack capacity  variables si.

Including buying variables yi,  slack variables si, and installed capacities bi, each arc constraint (2) yields:

aij,kxj,k+si−yi=bi		 i = 1, ..., m		(5)

- Figure II-3 – variable xi is the flow going through arc i; bi is the installed capacity; yi is the ammount of capacity to be installed

Those buying-variables have associated costs that are to be included in the objective function as follows:

MinimizeCjkXjk+ηiyi					(6)

They also have associated upper-bounds, that also should be included in the model. The complete model should then present the following matrix structure:

Figura II-4: Structure of the problem in the arc-path formulation including slacks and buying variables

## About M.C.M.F. Problem Size

Large telecommunications and transportation problems easily generate network models as large as 1000 nodes and 5000 arcs. Such networks may contain up to p  500.000 commodities ( =  (10,000 * 9999) / 2 ). Suppose only half of those commodities are really there - a common situation in real-world problems.

Both node-arc and arc-chain formulations result in large Linear Programming problems (LP):

- a node–arc formulation of such a network will result in close to 2.5 x 108 constraints ( = (250.000 x 1000) + 5000).

- an arc – chain formulation will result in 2.55 x 106 (5000 + 250.000).

The number of constraints in an LP problem defines the size of the working-basis that has to be kept inverted. However, GUB techniques reduce these numbers to much lower ones (see Lasdon [6]) .

Table 1

Size of the Matrix to be Inverted

| Algoritm | node - arc | arc - chain |
| --- | --- | --- |
| Simplex Method | 2.5 x 108 | 2.55 x 106 |
| GUB Method | 5000 | 5000 |

Anyway, a (5,000 x 5,000) floating-point  matrix (8 bytes per number)  would demand 200 Mbytes of RAM memory – still a large value. Thus, memory requirements are a n2 function of the number of arcs. Such matrix sizes are too large to be maintained in personal computers like the ones used in telecommunications or transportation engineering offices.

However, even larger single-commodity network flow problems are solved in such computers. Single commodity algorithms take advantage from tree-representations of the simplex basis, reducing the memory needs to a linear function of the number of arcs. The dream is to make the same with multi-commodity problems.

In this paper we describe one way to significantly reduce the amount of memory needed in multicommodity flow algorithm. We will show that it is possible to calculate dual and primal variables keeping an working-basis relatively small. Its size will be smaller than or equal to the number of arcs whose total flow reached the maximum, i.e. the number of saturated arcs.

In real world telecommunications / transportation problems, saturated arcs are usually less than 5%  of the number of arcs. In this case,  a network with 5000 would result in a 250 x 250 working basis, demanding 8 x 2,5 x 105 = 500 Kilobytes. This amount of memory  is found in small desktop computers.

However, there are two important features about this working-basis:

- Its size changes along the optimization process, since it is related to the amount of saturated arcs. How to update an inverted working-basis using pivoting matrix if its size changes?

- It is a very sparse matrix. How to take advantage from this sparsity ?

Using a product-form representation solves the two preceding questions simultaneously.

Let N be the number of nodes and let S be the number of saturated arcs in the network. Table 2 shows the different relative sizes of working-basis required by a traditional SIMPLEX, by a GUB basis-partitioning and by a cycle-matrix basis-partitioning .

Table 2

| Algoritm | node - arc | arc - chain |
| --- | --- | --- |
| Simplex | Arcs + (Nodes *Commodities) | Arcs + Commodities |
| GUB | Arcs | Arcs |
| GUB – CM | SaturatedArcs << Arcs | SaturatedArcs << Arcs |